The invention belongs to the field of military technology, specifically related to the activation of explosives with an electric detonator. It can operate as a detonator, but using an audible alarm, it can be applied in a civilian environment in the protection of people, objects or territories. The purpose of the invention is to create a safe multifunctional device with or without operator intervention, suitable for use in the destruction of manpower, armored vehicles and for training purposes or in the civilian field. The purpose of the invention is achieved by the fact that the housing (1) of the device for controlled detonation or alarm signalling is rectangular, made of weatherresistant materials, with an openable/closeable cover (2), inside of which is installed a replaceable rechargeable battery (3) with contacts (4), (5) at its ends and a microcircuit (6), with electronic components, which is connected to a group of light indicators (7) and an LED indicator (8) located on the housing (1), a detonation button (9), an electric circuit test button (10) intended for measuring the resistance of the electric detonator circuit, according to the value of which the corresponding number of LED indicators (7) lights up, and a power on button (11) is fixed on one of the side walls of the housing (1) and terminal release buttons (12) are installed on both side walls, which release the terminals (13) installed at one end of the housing (1), for inserting peeled off wires of electronic detonators or speaker wires, on the other end of the housing (1) there is a tumbler lever (14) and a spool for a line or cord (15).