The invention relates to measuring apparatus working by the principle of
high frequency electric (HFE) permittivity and can be used for measuring volumetric dampness by the dielectric constant in diverse materials.
A block of sensing  elements (1) includes a pair of identical condensers Cw1, Cw2, which have voltage Uf and Uf/n drop of constant relation by different frequency f and f/n, their electrodes stretched out into a plane are used as antenas A1, A2 of HFE and, owing to a block (2) of connection, are connected to a measuring bridge (8) alternately standard condensers Co1, Co2, a block of correction chains (10) is placed in front of an integrator (11) for correcting signals by the type of a measuring medium.