The invention relates to the branch of power, telecommunocations and an electricity supply. The purpose of the present invention is to upgrade an earthing, to increase reliability an earthing, to increase durability of a device, to improve service conditions.
The earting protection system comprises steel bars (1) covered with copper or zinc film the thickness of which is about 0,1-0,5 mm. The height of the bar is about 0,5-3,0 mm and the diameter of its is about 5-45 mm. The ends of the bars (1) are connected together with threaded sleeve (2) made from bronze. For hammering in the ground the head (3) of bar made from a reinforced steel is screwed in sleeve (2) and the steel clamp (4) is screwed on the bottom bar. For the connection the earthing device to a round (5) or flat (6) input the upper bar (1) is joined with a crossed flat tie or a crossed and shaped tie or an one-nut tie by standing the bar into the tie and fixing by nuts. The tie is covered with a concrete box with a cover. The earthing device may be comprised not more than 20 bars. The bars are hammered in the ground until the depth of about 25-30 m.