This invention relates to a process for the continuous boiling of wort, comprising feeding the unboiled wort to a wort heater, wherein it is heated to a temperature between 75 and 125 oC, introducing the heated wort in a plug flow reactor, preferably arotating disk holding column, followed by treating the wort obtained from said reactor in countercurrent with with steam in a stripping column.