A method of treating wastewater including: biologically treating wastewater under pressure; then reducing the pressure and substantially removing solids by dissolved gas flotation at the reduced pressure. A plant includes pressured biological treatment means (4) and pressure reduction means (7) constituting a dissolved gas flotation unit.