The invention relates to construction, in particular to devices for 
prospecting of the physical and mechanical characteristics of the soil and can be used for engineering geological exploration.
An apparatus for clipping the soil includes a working table, a clipping device with movable and stable parts, a ring for the soil, machinery of vertical and horizontal clipping powers, upper and lower dies.
New is that, a hollow disposed in the movable part of the clipping 
device is filled with thicked test soil and separated from it 
with a partition which has a sharp upper part, and its upper part matches
with the top of the movable part of the device.