The present invention relates to liquid nitrogeneous fertilizers.@The nutritional components ratio in liquid nitrogeneous fertilizers claimed is as follows (in mass %)@nitrogen			16-26@calcium oxide		 3- 6@magnium oxide		 2- 4@microelements		0,01-1,00 and@water				the rest to 100.@The raw material is dolomite which is subjected to decomposition for 10-90 minutes at the temperature 20-100 °C with nitric acid of concentration  20-58 % , neutralization with ammonia or ammonia water to pH 1,5-7,5, standartization with urea or other nitrogen component in amount of 0,1 to 560 mass % of total amountof dolomite in the form of solution in water or fusion.@The amount of microelements in the liquid nitrogeneous fertilizers is increased by adding copper, cobalt manganese sulphates, chlorides, carbonates, nitrates, oxides, chelated derivatives and molibdates; boric acid, borates, boron minerals, selenates or iodides.