The invention relates to the electrochemistry, especially to hand devices for electrochemical treatment of water and water solutions. Hand device for electrochemical treatment of liquids comprises cylindrical frame made from dielectric material with a lid, a membrane fixed device made from dielectric material and placed in a frame, an anode and a cathode, which are fixed in a lid. A membrane fixed device is made from hollow thin - walled cylinder with a bottom, which cross - section has a semicircle form, a side wall of a frame and a bottom comprises devices in which plane membrane is disposed. A height of a membrane fixed device with placed membrane is equal or smaller to a height of a frame. A volume of it is 0.4 - 05 of a frame volume. A pair of additional silver electrodes can be fixed on a lid.