The invention relates to the nuclear industry. Accident prevention and transposition automatic and manual servomotor for absorbing stick of RBMK reactor can be used for control of nuclear reaction by  absorbing sticks. A servomotor includes an electric motor (1) with electromagnetic broke, a shaft of which by diminishing reducer (2) of a drive is connected to electromagnetic joining muff (3) with a thermister (20), and by it and transitional reducer (4) is connected to a shaft (5) of reel, which is connected to hydraulic broke (6), which comprises a gear pump (7) and a baffle (8). A shaft is connected by safety muff (14) with selsyn reducer (15), selsyn (16), a cam - gear (17) and a micro - switch (18). A shaft (5) is connected to removable manual mechanism (12) for transposition a position. Parts of electromagnetic joining muff (3) are disposed separate from an electric motor, a bracing mechanism, selsyn, a micro - switches. End join of hydraulic brake (6) to shaft of a gear pump (7) is made cross muff - shaped.