The invention relates to devices for measuring of linear dimensions and can be used in precision length measuring machines, in calibration comparators of reference and maximum accuracy dash line and measures. A carriage of a device comprises two compound carriages, which are turned in respect of each other at a 180° angle. For that purpose bearings of carriages are arranged in isosceles triangle manner. Compound carriages with bearings can be disposed in the same section of a guide. A guide can be with a symmetric cross – section. Such guide is mere shot, simple, more stable and precise. Power and precise attachments are fixed to separate compound carriages and an interlink with a vibration silencer disposed in that gear power is transmit only in direction of measuring motion and passes over mass center of carriages.