Isolating  ball valve, comprising  body (1) with a coaxial inlet and outlet openings (8) and shut-off ball (2) engaged on the axis perpendicular to the axis of the openings (8), characterized in that  the shut-off ball (2) is embedded inwards symmetrically with respect to the axis of rotation of the shut-off ball (2) with movable pistons (3), which exterior surfaces  are a subset of the surface of  the shut-off ball  (2), between which pistons the bolt (4) is located  being of other than circular shape in cross-section  perpendicular to the axis of rotation, and  which axis of rotation being coaxial with the axis of rotation of shut-off ball (2), wherein a pistons (3) on the bolt (4) side have the stringy elements (6), arranged symmetrically to the displacement axis of the pistons.