The invention relates to devices for electric power generation installations using the piezoelectric effect. The invention aims piezoelectric vibration generator of electricity produced and increase the operating frequency range, the generator excitation in a wide frequency range varying subwoofers periodic or random signals. This goal is reached that in piezoelectric generator of low-frequency vibrations of the conversion into electrical energy comprises in a frame arranged resonator with concentrated mass mounted on the end of bracket and  at piezoelectric cantilever type actuator with concentrated mass mounted on a bracket end, at least one additional  resonator is arranged in construction, a natural frequency of resonator satisfies the condition fk = n × f (k-1), where n - an integer,  f - a resonator, the natural frequency (Hz), k - the resonator number (2? k? j, j - total cavity number) .