The invention relates to the construction of a panel house, more precisely, to the production of wooden panels with straw heat-insulating fillers. The proposed method for the production of shield with straw filling differs by the fact that a straw workpiece (5) is prepared industrially, for which can be used straw rolls as a raw materials, straw bundles or loose straw. These raw straw derivatives  are unwrapped, lacerated, dried and from it formed straw workpiece (5) of desired diameter, which are placed in a shield, pressed to the desired density and secured in such a position. Proposed rectangular, trapezoidal and curved forms of shields for home walls and vaults. Lining and overhead panels are also offered. Proposed technology line for the production of a panel with a straw filler, which has panels assembly conveyor (9) and a press (20) roller unwrapping and breaking mechanism (6), conveyor (7) of forming workpieces, and transfer device (8) of workpieces, a drying device. There are described in detail of the structure of the roll unrolling and breaking mechanism (6), of the workpieces forming conveyor (7) and of the workpieces transfer device (8).