The invention relates to the field of medical technology for physiotherapy devices used for body weight negative local physiotherapy or millimeter microwave stimulation. The device consists of power supply control panels 1 and microwave head (s) 9. Power supply and control panel 1 consists of power supplies with voltage regulator 2, with power indicator 3, with microwave head connection unit 6 and with microwave head assembly wires 8; the control units are placed in sensitive liquid crystal display (TFT LCD) 4 with a single crystal microprocessor 5 and external memories connected by a connector 7. The microwave head (es) consists of a housing 16, a waveguide 13 with a microwave generating diode 12, power and control connections 10, head identification and control unit 11 and head identification code 14, operating indicator 15. Patented microwave physiotherapy device shapes and emitted modulated microwaves in the 42.0-44.36; 52.5-55.4; 63.0-66.54 GHz bands. The device has from 1 to 10 microwaves 9.