A patented composition is granular compound fertilizer with a microbiological preparation and its production method, in which the mixture of fertilizer components is moistened with acidified water in a mixer, heated with water vapor, the microbiological component is fed to the back of the mixer, then the mixture is fed to the granulator, then dried in a dryer, fractionated in the first sieve, the small part of the granulated product is returned to the mixer of the initial mixture, the rest is cooled in the cooler, fractionated in the second sieve, the frontl part is delivered to the warehouse, the small part is returned to the original mixer in the second sieve, the large part is comminuted in a shredder and fractionated again in a second sieve. Bacillus sp bacteria are used as a microbiological preparation.