The invention discloses a filler wire feeding device for TIG welding of metals having a gun-shaped holding housing (1) in which are mounted a wire feed guide tube (3), a wire feed mechanism having a servo motor (13), a pulley (14) mounted on the servo motor (13) gearbox output shaft (15), and a wire feed wheel (16) with a resilient working surface interacting with the wire rod (2) through a notch (17) in the wire feed guide tube (3), a wire clamping mechanism consisting of two plates metal springs (7) inserted in two notches (8) in the wall of the wire guide tube (3), and two balls (9) and (10) slidably inserted in the round holes (11) in the bottom of the notches (8) in contact with an inner cavity (12) of the wire feed guide tube (3), a switch (24) mounted on the handle (22) of the support housing (1), and a power supply unit (23) with rechargeable elements inside the handle (22) of the support housing (1).