The patent relates to an efficient welding device for the production of RV reducer shell, which comprises a base. The top of the base is fixedly provided with a protective box, and one side of the top of the protective box is fixedly provided with a driving motor, which can control the left and right movement of the splint by rotating the handle of the threaded rod, and the splint can clamp and fix the reducer shell placed on the welding table to facilitate the welding operation. The drive motor can control the screw clockwise or counterclockwise rotation, the screw rotation can control the slider sliding around, and then can control the welding head moving around, the electric telescopic rod can control the welding head moving down, and the welding head can be welded on the splint clamping fixed reducer shell. The exhaust fan can suck the flue gas produced during welding from the protective box cavity through the suction cover and then exhaust it into the industrial exhaust gas purifier cavity through the pipeline. The industrial exhaust gas purifier can purify the toxic and harmful substances in the flue gas, which can prevent the flue gas from being inhaled by the staff.