The disclosure discloses an automatic welding equipment for a suspended plate anti-explosion door leaf, including a first stand column and a second stand column which are left-right symmetrical, and a placement board rotatably connected to and between the first stand column and the second stand column, and further includes a first drive assembly arranged on the first stand column to drive the placement board, a first opening opened in the center of the top of the placement board, two second openings left-right symmetrically opened on the top of the placement board, positioning and clamping mechanisms arranged in each of the two second openings to fix the suspended plate anti-explosion door leaf, and a second driving assembly arranged in the first opening to drive the movement of the positioning and clamping mechanisms. Through this equipment, it is convenient to adjust the distance of the two positioning and clamping mechanisms, so as to fix the door leaves of different specification and improve the application scope and production efficiency.