The invention is in the category of special-purpose locking devices. The protective locking device for a truck fuel tank is comprised of a plastic stopper (1), consisting of an upper part covering the opening of the fuel tank with an ergonomic cone-shaped stainless steel protective cap (2), and a lower part recessed into the filler neck of the fuel tank, consisting of a gasket (3) and a moulded slot for a metal cross-piece (4). A central vertical channel is formed in the inner part of the plastic stopper (1), which is fitted with a metal central tube (5) with an outer ring in the upper part, which is plugged by a rubber stopper (6) from above. A locking bolt (10) is loosely inserted into the metal central tube (5) from below and threaded through the threaded hole in the metal cross-piece (4) and secured by means of a washer (12) placed at its free end under the metal cross-piece (4), which is secured by means of a clip (13) through a hole in the free end of the locking bolt (10).