Wheel fastening system with belts, preferably for the transport of semi-trailers, trailers and / or motor vehicles. The belt fastening system has a perforated sheet plate 1 with deeply drawn oval holes 2, which are oriented in the longitudinal direction of the plate (freight) and can be arranged in a checkerboard pattern. Optimal hole diameter in the longitudinal direction 29 ÷ 30 mm, in the horizontal direction 18 ÷ 19 mm. At least one conical wheel brake wedge 3a and / or 3b is fastened in the holes 2 of the plate 1, the latter having a locking leg 11 rigidly fixed at one end of the housing, which is locked in the hole 2 in the plate in the working position by turning the housing. At the other end of the housing it has a handle 7, which, when turned, locks the cam 10 in the working position. The belt 5 is fastened at three points 1A, 1B, 1C of the plate, one of which may be a means of receiving the fastening element 4a of the belt 5a, the other being fastened between the wheel and the outgoing part 5b, the fastening element 4b being fastened in the nearest hole 2 the third fastening element 4c is fastened through the tensioning latch 6 in the nearest hole 2 of the plate 1 opposite to the exit direction of the belt 5b.