The invention relates to a multilayer composite panel with a central filler made of devulcanized rubber or hybrids obtained on its basis, and more particularly to a three-layer composite panel consisting of two outer reinforcing composite sheets and a central filler of devulcanized rubber or hybrids obtained on its basis. The whole three-layer composite board is obtained by joining reinforcing composite sheets and internal devulcanised rubber filler together by heating, lamination, ultrasonic welding, friction welding, laser, high frequency current, heated air or heated combustion gas (methane, butane), infrared radiation and etc. The three-layer composite panel can be used in industry both as a separate structural element and as a composite element for the production of various products, such as pallets, partitions, building partitions, sound barriers.