The claimed invention refers to the lifting and transport equipment used to move the basket with a car semi-trailer. The traverse for moving the basket with a car semi-trailer comprises longitudinal beams 1, transverse beams 2, on the beams there are nests 3 for placing hook suspensions 4 in them and fittings 5 for connection with the spread mechanism of the reachstacker, longitudinal 1 and transverse beams 2 form a frame, characterized in that the fittings 5 for connection with the spread mechanism of the reachstacker are placed on the transverse beams 2.