The invention is intended for ventilation devices and relates to the construction of flush-mounted diffuser connection boxes for connecting air diffusers to ducts of various diameters and shapes and can be adapted and used for air distributor boxes of various shapes and installation types. The invention includes the construction of a diffuser connection box having an opening (1) for the base of the diffuser (2) with a hole (3) for air flow and fixed plaster elements (4), in which at least one of the caps (6) covering the ends of the box body (5) has an extension side walls (7) and pulled out increases the length of the box opening (1) and makes it possible to install an inverted diffuser base (2) with gypsum or other easily scrubbed material elements (4) directed inside the box. The invention also includes methods of mounting the diffuser base (2) in the opening of the box (1), when the plaster elements (4) are located inside the box and become protected from contact with external objects during transportation or storage.