The invention belongs to field of agriculture in particular veterinary medicine. The system is designed to induce mechanical vibrations in udder of cows through teats thus activating blood circulation which affects inflammatory processes in udder – cow mastitis. The system comprises at least one milking device on which a vibrating stimulus drive is rigidly mounted in a housing which is connected to a controller controlled via a vacuum feed converter. The system preferably comprises four separate vibrostimulating elements, each mounted on four separate milking devices. All vibrostimulators are connected to a single controller controlled by a vacuum feed converter.