The frame submitted for registration is intended to rigidly attach the boat canopy to the corresponding outer part of the boat hull. The design of the frame allows the awning to be fastened quickly and securely in unexpected weather conditions which is especially important in recreation passenger boats. The shape of the frame adapts to the shape of the hull can also be assembled from individual parts. The shape of the frame profile is a plane (2) with both bent edges facing one side and parallel, the bending edge of the upper edge (3) is rounded, the opening angle of the lower edge (4) is in the range of 45°÷70°, the optimal angle is 47°÷50° in the horizontal part of this edge (4) there are openings (7) for attaching the frame to the hull (14). On the stem edge (9) in which the frame (1) is fastened hooks (10) are mounted, on which the tension rope (12) of the canopy is fastened.