The present invention discloses a system for attaching objects to surfaces by means of waterproof through-hole fastening, providing long-term reliable waterproofing of the hole. For example, a roof surface in which a hole has been drilled for fixing solar modules or other structural elements to the roof surface. The invention includes the following essential elements: a convex rigid washer or nut having a central convexity around the central bore and a flat peripheral portion which rests against the mounting surface. Beneath this washer/nut convexity is a low-pressure elastic gasket, which is pressed against the bore in a fixed, limited volume of the gasket's convexity, thus ensuring waterproofing of the bore around the bolt. And in the peripheral part of the washer, a high-pressure gasket is used, resistant to heavy loads, waterproofing the gap between the mounting surface and the gasket, and capable of sustaining heavy loads, made of, for example, EPDM rubber. In this way, the elastic central gasket reliably waterproofs the hole and does not experience unwanted loads.