The present invention is related to the field of fertilizer production and can be applied in the production of sapropel based organic fertilizers, dedicated to the improvement of the beneficial agronomic and biological properties of the soil, or mixtures of organic and mineral fertilizers and ameliorants. Sapropel based organic fertilizers, consists of dewatered sapropel, crushed straw, crushed high bog peat and water-absorbing hydrogel, and one tone of standard mineral nitrogen fertilizers contains: a) 450 kg (45%) of dry matter, from which 112 kg (25%) of dry long-life matter from sapropel and 264 kg (59 %) of crushed straw, 63 kg (14%) of dry matter from peat, 8 kg (1,8%) ofhydrogel water-absorbing in large quantities (1:250), 3 kg standard mineral nitrogen fertilizers; b) 550 kg (55%) of water. The method for producing sapropel­ based organic fertilizers includes dewatering natural sapropel by gravity in technological bags, mixing it in a mixer with crushed straw, crushed high bog peat, hydrogel and standard mineral nitrogen fertilizers when the mass ratio of the working technological mixing mixture in kilograms (error limits+/- 5%) is 800:150:45:4:1 and additionally drying in the technological site to a total moisture content of 55% of standardized fertilizers. In addition, biological preparations that accelerate the decomposition of straw are introduced.