The invention relates to exterior window sills of operational residential houses or buildings of other purpose. A window sill made of sheet metal has an inclined plane (6), an upwardly inclined front edge (7) attached to the lower part of the window frame (8) and a downwardly bent outer edge (9) that protrudes beyond the building wall (10). The novelty of the invention is that an artificial grass cover tape (11) is glued on the inclined plane (6) using a sealant based on synthetic rubber for roof and facade sealing and repair work, wherein the thickness d of the artificial grass cover tape (11) is 7–12 mm and the colour is matched to the colour of the outer edge (9) of the inclined plane (6).