The invention belongs to the field of logistics, designed to facilitate the transportation and storage of goods. The top plate of the cardboard pallet consists of a base plate and a multilayer corrugated cardboard sheet glued on it. The base plate (1) is formed from corrugated cardboard strips (6) glued with each other, which are cut from a block of multi-layer corrugated cardboard sheets (5). The cut strips are glued together on the sides to form the long edge of the tray. The pallet has three lower supporting strips of cardboard with glued hollow supports, or six lower supporting strips with glued hollow cardboard supports between them and connected to the upper plate of the pallet by gluing. When manufacturing a cardboard pallet, first of all, a block (5) is formed from sheets of multi-layer corrugated cardboard, from which strips (6) are cut so that the tops of the corrugations are would be perpendicular to the plane of plate. A multi-layer corrugated board block is made by pressing together perforated corrugated board sheets, which are first sprayed with hot glue and hot compressed air on both sides, and then the block is repeatedly hot air-treated and cut. A glued pallet is impregnated, bandaged into blocks and matured.