The technical field of the invention is ventilation systems. A technical problem being solved - fixing linear slit diffusers to a ceiling. The invention is a method of fixing an integrated linear ventilation slit diffuser, which includes fixing the diffuser to the ceiling construction elements and stabilizing the facade panel of the integrated linear ventilation slit diffuser between and to the gypsum board mounting profiles and fixing it with fasteners. The diffuser is attached directly to the prepared ceiling profiles (they are already adjusted to the required height) - the diffuser is screwed to the profiles in the required place with the same fastening elements as plasterboards. The mounting method of the integrated linear ventilation slit diffuser allows one worker to perform the installation work. It also provides an opportunity to connect several segments of diffusers very quickly, without having to adjust the position of the diffusers hanging in the air with two projections. There is also no need to attach to the concrete ceiling, no need to select any fasteners.