The invention relates to ventilation linear slit diffusers. The technical problem being solved is the connection of plastered linear diffusers. In one aspect, the invention discloses a linear diffuser comprising a faceplate (FP) and a diffuser junction box (2). The faceplate of the diffuser comprises at least two segments (1.11, 1.12) and a longitudinal gap (1.2) between the segments (1.11, 1.12) which are provided parallel to each other, which form a pair of segments. According to the second aspect, the method of connecting the linear diffusers includes positioning the end of the first segment (1.11) with the first connecting surface (1.8) adjacent to the end of the second segment (1.12) with the second connecting surface (1.9), and the end of the first segment (1.11) with the second with the connecting surface (1.9) positioned adjacent to the end of the second segment (1.12) of the pair with the first connecting surface (1.8), where at least one additional segment (1.13, 1.14) is attached to each segment of the pair so that along each segment ( 1.11, 1.12, 1.13, 1.14) the first connecting surface (1.8) would come into contact with the second connecting surface (1.9) of the further segment, when the protrusion (1.91) is pushed into the socket (1.81) or the second connecting surface (1.9) would come into contact with the first connecting surface of the further segment ( 1.8), reducing the protrusion (1.91) to the socket (1.81).