The composition of fly ash Portland cement is obtained using a high-purity sulfoaluminate additive as a binder and hardening regulator. The additive contains >97,5 % yelmite. It is obtained from calcium carbonate, aluminum hydroxide and gypsum by hydrothermal and solid phase synthesis. This additive is high purity, environmentally friendly, non-toxic, alkali-free and suitable for fly ash Portland cement.