The household biohumus composter belongs to the composting equipment. It can be used to compost biodegradable waste using California earthworms and produce biohumus fertilizer. The composting cylinder consists of the following parts: two plastic smps (1) and (2) insulation material between them (3) a bottom (6) with insulation material protecting from temperature differences and a metal mesh (12) protecting from rodents, a ventilation hole with a metal mesh (7) used for biohumus extraction and ventilation, a top cover with a plastic strip (13) and a metal mesh (14) used forflie protection and natural precipitation gathering, a drainage pipe (8) with a plastic container (9) for excess water gathering. In the cold season, the ventilation hole is closed with insulation material (17). The top cover is replaced with an OSB board (4) with foam (5) used for positive temperature support. The best time to extract biohumus is in spring or autumn.